Extra low rise electrohydraulic lift with min. travel 96cm and double platform. Platforms connected by double torsion beam for perfect synchronicity and levelling.

Double redundant hydraulic circuit with 4 pistons, 2 on each platform, with safety and shut-off valve on the hydraulic circuit for overload protection and hoses breaks. Self-lubricating axle bearings on each rotating pin, no maintenance required, High- sliding and self-lubricating polyethylene sliding pads. Lowering speed control with compensation valve. “Hold to run” 24V Control.

Electric Micro Switch with “foot guard” device and acoustic signal, to protect the last lowering travel. Suitable for both floor and recessed installation, even under difficult conditions in case of not available deep pits.